# AI Signal Processing Tutor

Capstone Project — ECE Students

Project question: How can AI explain signal-processing concepts and generate Python code that is verified against known results?

# 1. Objectives

Explain six core topics at beginner and advanced levels.

Generate one reproducible Python demonstration per topic.

Verify formulas, numerical outputs and plots against expected theory.

Compare three AI tools and two prompting strategies using recorded evidence.

Create a quiz, study guide, presentation and prompt library.

Explain responsible AI use and limitations.

# 2. Topic explanations

## Sampling

Beginner and advanced explanation: Sampling converts a continuous-time signal x(t) into a discrete sequence x[n] = x(nTs), where Ts = 1/fs. For a band-limited signal with maximum frequency B, ideal reconstruction requires fs > 2B. Example: an audio signal is sampled before digital storage.

Key check: a 5 Hz signal sampled at 50 Hz for 1 s with endpoint excluded produces 50 samples.

## Aliasing

Beginner and advanced explanation: Aliasing occurs when the sampling rate is too low to distinguish different input frequencies. Frequencies separated by integer multiples of fs can produce the same samples. For a real sinusoid, an alias can be folded into the interval from 0 to fs/2.

Key check: a 9 Hz sinusoid sampled at 12 Hz has an alias at |9 - 12| = 3 Hz.

## FFT

Beginner and advanced explanation: The discrete Fourier transform expresses a finite sequence as frequency components. The FFT is an efficient algorithm for computing the DFT. For N samples at sampling rate fs, frequency bins are spaced by fs/N.

Key check: a 1 s record sampled at 100 Hz has 1 Hz bin spacing; tones at 5 and 15 Hz appear at those bins.

## Low-pass filtering

Beginner and advanced explanation: A low-pass filter passes lower frequencies and attenuates higher frequencies. This demonstration uses a 9-point moving average, a simple FIR filter. Its smoothing effect comes with a trade-off: it can attenuate or distort desired signal components too.

Key check: compare noisy input, filtered output and clean reference; quantify error rather than judging only by appearance.

## Convolution

Beginner and advanced explanation: Discrete convolution combines sequences: y[n] = sum_k x[k] h[n-k]. It is central to the response of linear time-invariant systems. The full convolution length is Nx + Nh - 1.

Key check: [1,2,3] convolved with [1,1] gives [1,3,5,3].

## Noise

Beginner and advanced explanation: Noise is an unwanted random or interfering component added to a signal. In the demo, Gaussian noise with standard deviation 0.3 is added to a sine wave. A single finite random sample may not have exactly that standard deviation.

Key check: use a fixed random seed for reproducibility and compare measured noise standard deviation to the target with a tolerance.

# 3. Verification procedure

Run signal_processing_demos.py in Python or upload the notebook to Google Colab.

Check that the script exits with return code 0 and all assertions pass.

Inspect each plot for correct axes, units, legends and expected features.

Compare theoretical and observed values using the verification report.

Record the actual Python/NumPy versions, date, any errors, and corrections made.

# 4. AI tools and prompting strategies

Suggested tools: ChatGPT, Google Gemini, Claude, Google Colab and Canva. Use the same task across the three AI assistants. Strategy A is a short general prompt; Strategy B explicitly requests formulas, test cases, expected outputs, edge cases and independent verification. Fill the comparison table only after performing the tests.

| Criterion | ChatGPT | Gemini | Claude |
| --- | --- | --- | --- |
| Formula correctness | Complete after testing | Complete after testing | Complete after testing |
| Code execution | Complete after testing | Complete after testing | Complete after testing |
| Plot accuracy | Complete after testing | Complete after testing | Complete after testing |
| Clarity | Complete after testing | Complete after testing | Complete after testing |
| Verification quality | Complete after testing | Complete after testing | Complete after testing |

# 5. Before/after analysis

Before verification, AI output should be treated as a hypothesis, not a result. After verification, each topic should have a runnable example, stated assumptions, numerical checks and plots compared with theory. Document actual errors found; if no errors occurred, state that no error was observed in the tests performed.

# 6. Responsible AI use and limitations

AI may hallucinate references or produce plausible but incorrect equations.

A graph can look reasonable while its units, frequency axis or normalization are wrong.

Code that runs is not automatically scientifically correct.

Use trusted textbooks and official documentation, and independently calculate key results.

Disclose which tools were used and retain prompts and evidence.

Do not invent tests, tool comparisons, errors, or verification claims.

# 7. References

A. V. Oppenheim and R. W. Schafer, Discrete-Time Signal Processing, Pearson.

A. V. Oppenheim, A. S. Willsky and S. H. Nawab, Signals and Systems, Pearson.

NumPy documentation — Fourier Transforms: https://numpy.org/doc/stable/reference/routines.fft.html

NumPy documentation — Convolution: https://numpy.org/doc/stable/reference/generated/numpy.convolve.html

Matplotlib documentation: https://matplotlib.org/stable/